| Engine | Type | 3Y-E | 3Y | 2L-T | 2L |
| --- | --- | --- | --- | --- | --- |
|  | Type | Water cooling serial 4 cylinders OHV | Water cooling serial 4 cylinders OHV | Water cooling serial 4 cylinder OHC turbos | Water cooling serial 4 cylinder OHC turbos |
|  | Use fuel | Unleaded gasoline | Unleaded gasoline | Gas oil | Gas oil |
|  | Total cubic displacement cc | 1.998 | 1.998 | 2.446 | 2.446 |
|  | Inside diameter × major diameter mm×mm | 86.0×86.0 | 86.0×86.0 | 92.0×92.0 | 92.0×92.0 |
|  | Compression ratio | 8.8 | 8.8 | 20.0 | 22.3 |
|  | Installed capacity ps/r.p.m | Net 97/4.800 | Net 88/4.800 | Net 85/4.000 | Net 76/4.000 (surf) Gross 83/4.000 (pickup) |
|  | Largest torque kg-m/r.p.m | 16.3/3.800 | 15.8/3.400 | 19.2/2.400 | Net 15.7/2.400 (surf) Gross 17.0/2.400 (pickup) |
|  | Fuel feeder | EFI | Carburetor | Bosch type distribution type (jet pump) | Bosch type distribution type (jet pump) |
|  | Quantity inside fuel tank | 65 | 65 | 65 | 65 |
| Steering wheel; Ball & nut system. |
| Suspension; Before. After. | ダブルウイッシ�…ボーントーションバー (as for pickup, semi- elliptic leaf spring) |  |  |  |
|  |  | Semi- elliptic leaf spring |  |  |  |
| Brake; Before. After. | Disk |  |  |  |
|  |  | Leading trailing (as for pickup duo servo) |  |  |  |