DRIVE TRAIN

Rear diff.....1.9L Hypoid gear oil API GL-5 (LSD oil if you have a Limited Slip Diff), SAE 90.

Front diff....2.2L Hypoid gear oil API GL-5 ,SAE 75W-90.

Manual Gearbox......3.0L Multipurpose Gear oil API GL-4 or GL-5, SAE 75W-90

Automatic Gearbox......Up to 4.5L Dexron II ATF fluid.

Transfer box..1.1L Multipurpose Gear oil API GL-4 or GL5, SAE 75W-90.

Wheel bearings....Lithium base wheel bearing grease NLGI No.2

Propshafts....

Spiders and slide yokes..Lithium base chassis grease NLGI No.2

Double cardan joint......Molydbenum-disulfide lithium base chassis grease NLGI No.2

Ball joints......Molydbenum-disulfide lithium base chassis grease NLGI No.1 or No.2

Front drive shaft drive bushings....Molydbenum-disulfide lithium base chassis grease NLGI No.2

Clutch fluid...SAE J1703 or FMVSS No.116 DOT 3

BRAKES

Minimum pedal floor clearance when depressed...60mm
Pedal freeplay.....3-6mm
Parking brake adjustment.....13-19 clicks
Brake fluid...SAE J1703 or FMVSS No.116 DOT 3

STEERING

Wheel free play....>30mm
Power Steering fluid..... ATF fluid Dextron-II.

TYRES

265/70 R15 110S....
Front 29psi, rear 29psi Loaded or unloaded
215 R15 100S....
Front 28psi, rear 33psi Loaded or unloaded

ENGINE

3VZ-E
Engine oil.........4.2L (4.5 inc filter)

1KZ-T
Engine oil.........7L (8L inc filter)

A102 Oil Filter Toyota Part No. T90915-30002 (By Mart)

2L-T
Engine oil.........5.8L (6.8 inc filter)

Oil Grade.
Petrol.....
API SG or SH multigrade, or ILSAC multigrade. 
Recommended viscosity....
Diesel.....
API CD or better

Fault Codes
Diagnostic fault codes for the 2LTE diesel engine. 

1 - Normal 
2 - Turbo boost pressure sensor 
4 - Coolant temperature sensor 
5 - Correction resistance (Pump) 
6 - Pump speed sensor (Pump) 
7 - Throttle position sensor 
8 - Intake air temperature sensor 
9 - Vehicle speed sensor 
10 - Starter switch 
11 - Switch sign (Vehicle not in P or N or A/C on) 
12 - Timer control valve (Pump) 
13 - Crank position sensor 

Vacuum pump rotors......Toyota Part 29331-56070 and cost £13.15 +VAT each.
'89-'91 cars use four and the '91-'94 cars with the later type of alternator use three. thanks to Elvie

Cooling........

3VZ-E.....10L (9.8 with Auto)
2L-T......9.1L
1KZ-T.....10L

Type......Ethylene-glycol antifeeze/anticorrosive

Thermostat (91' 2.4)..... 88 degree (Toyota part no 16340 54010) thanks to JimL
Cap (91' 2.4)..... 0.9 bar (Toyotoa part no 16401 63010) thanks to JimL



Cylinder Head............


2.4 2LT/E... thanks to Koi
1st stage 58ft/lb 78nm
2nd stage 90 degree turn 
3rd stage 90 degree turn.